REQUIRED SWPPP COMPONENTS

In accordance with the General VPDES Permit for Discharges of Stormwater from Construction Activities, Article IV of the Prince William County Stormwater Management Code, and Section 700 of the Prince William County Design and Construction Standards Manual, the SWPPP must include the following information:

- General Information

a. 	A signed copy of the registration statement for coverage under the CGP;

b. 	Upon receipt, a copy of the notice of coverage under the CGP (i.e., notice of coverage letter);

c. 	Upon receipt, a copy of the CGP;

d. 	A narrative description of the nature of the construction activity, including the function of the project (e.g., low density residential, shopping mall, highway, etc.);

e. 	A legible site plan identifying:

- Directions of stormwater flow and approximate slopes anticipated after major grading activities;

- Limits of land disturbance including steep slopes and natural buffers around surface waters that will not be disturbed;

- Locations of major structural and nonstructural control measures including sediment basins and traps, perimeter dikes, sediment barriers, and other measures intended to filter, settle, or similarly treat sediment that will be installed between disturbed areas and the undisturbed vegetated areas, in order to increase sediment removal and maximize stormwater infiltration;

- Locations of surface waters;

- Locations where concentrated stormwater is discharged;

- Locations of support activities, when applicable and when required by the VSMP authority, including but not limited to: (i) areas where equipment and vehicle washing, wheel wash water, and other wash water is to occur; (ii) storage areas for chemicals such as acids, fuels, fertilizers and other lawn care chemicals; (iii) concrete wash out areas; (iv) vehicle fueling and maintenance areas; (v) sanitary waste facilities, including those temporarily placed on the construction site; and (vi) construction waste storage; and

- When applicable, the location of the on-site rain gauge, or the methodology established in consultation with the VSMP authority, used to identify measurable storm events for inspection purposes.

- An Erosion and Sediment Control (ESC) Plan developed and approved in accordance with Section 750.00 of the Prince William County Design and Construction Standards Manual, and Administrative Procedures Manual.

- A Stormwater Management (SWM) Plan developed and approved in accordance with Section 722.02 of the Prince William County Design and Construction Standards Manual and Administrative Procedures Manual .

- A Pollution Prevention Plan (PPP) developed and implemented in accordance with Section 750.06 of the Prince William County Design and Construction Standards Manual and Administrative Procedures Manual.

- Information demonstrating compliance with the Total Maximum Daily Load (TMDL) Requirements per Article IV, Section 23.20 of the Prince William County Stormwater Management Code.

- Qualified personnel responsible for conducting on-site and off-site inspections required by the CGP shall be identified by the Operator in the SWPPP.  The Operator is responsible for insuring that the qualified personnel conduct inspections.

- The SWPPP must identify individuals or positions with delegated authority to sign inspection reports and/or amend or modify the SWPPP.

- The SWPPP shall be signed and dated in accordance with the signatory requirements.

- The SWPPP must contain documentation of all amendments, modifications, and updates.

- The SWPPP must include documentation of inspections conducted in accordance with the inspection requirements.

- The SWPPP must contain documentation of corrective actions taken in accordance with the requirements.

| REQUIRED SWPPP COMPONENTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Required Components | Permit Reference | Other References | Responsible Party | Location |
| A signed copy of the registration statement for coverage under the CGP | Part II A.1.a |  |  | Tab15 of the SWPPP Binder |
| A copy of the notice of coverage under the CGP | Part II A.1.b |  |  | Tab 16 of the SWPPP Binder |
| A copy of the CGP | Part II A.1.c |  |  | Tab 17 of the SWPPP Binder |
| A narrative description of the nature of the construction activity, including the function of the project | Part II A.1.d |  |  | Tab 4 of the SWPPP Binder |
| A legible site plan identifying: |  |  |  |  |
| Directions of stormwater flow and approximate slopes anticipated after major grading activities | Part II A.1.e.(1) |  |  | Sheet __ of the SDP |
| Limits of land disturbance including steep slopes and natural buffers around surface waters that will not be disturbed | Part II A.1.e.(2) |  |  | Sheet __ of the SDP |
| Locations of major structural and nonstructural control measures including sediment basins and traps, perimeter dikes, sediment barriers, and other measures intended to filter, settle, or similarly treat sediment that will be installed between disturbed areas and the undisturbed vegetated areas, in order to increase sediment removal and maximize stormwater infiltration | Part II A.1.e.(3) |  |  | Sheet __ of the SDP |
| Locations of surface waters | Part II A.1.e.(4) |  |  | Sheet __ of the SDP |
| Locations where concentrated stormwater is discharged | Part II A.1.e.(5) |  |  | Sheet __ of the SDP |
| Locations of support activities, when applicable and when required by the VSMP authority, including but not limited to: (i) areas where equipment and vehicle washing, wheel wash water, and other wash water is to occur; (ii) storage areas for chemicals such as acids, fuels, fertilizers and other lawn care chemicals; (iii) concrete wash out areas; (iv) vehicle fueling and maintenance areas; (v) sanitary waste facilities, including those temporarily placed on the construction site; and (vi) construction waste storage | Part II A.1.e.(6) |  |  | Sheet __ of the SDP |
| When applicable, the location of the on-site rain gauge, or the methodology established in consultation with the VSMP authority, used to identify measurable storm events for inspection purposes | Part II A.1.e.(7) |  |  | Sheet __ of the SDP |
| An Erosion and Sediment Control (ESC) Plan | Part II A.2 |  |  | Sheet __ of the SDP Tab 7 of the SWPPP Binder |
| A Stormwater Management (SWM) Plan | Part II A.3 |  |  | Sheet __ of the SDP Tab 7 of the SWPPP Binder |
| A Pollution Prevention Plan (PPP) | Part II A.4 |  |  | Sheet __ of the SDP Tabs 7 and 8 of the SWPPP Binder |
| Information demonstrating compliance with the Total Maximum Daily Load (TMDL) Requirements | Part II A.5.a |  |  | Tab 9 of the SWPPP Binder Sheet __ of the SDP |
| Qualified personnel responsible for conducting on-site and off-site inspections required by the CGP shall be identified by the Operator in the SWPPP. | Part II A.6 |  |  | Tab 6 of the SWPPP Binder |
| The SWPPP must identify individuals or positions with delegated authority to sign inspection reports and/or amend or modify the SWPPP. | Part II A.7 |  |  | Title Sheet and Tab 5 of the SWPPP Binder |
| The SWPPP shall be signed and dated in accordance with the signatory requirements | Part II A.8 |  |  | Title Sheet of the SWPPP Binder |
| The SWPPP must contain documentation of all amendments, modifications, and updates | Part II B |  |  | Tab 10 of the SWPPP Binder |
| Upon commencement of land disturbance, the operator shall post conspicuously a copy of the notice of coverage letter near the main entrance of the construction activity. | Part II C |  |  | Tab 11 of the SWPPP Binder |
| The SWPPP must include documentation of inspections conducted in accordance with the inspection requirements | Part II F |  |  | Tab 12 of the SWPPP Binder |
| The SWPPP must contain documentation of corrective actions taken | Part II G |  |  | Tab 13 of the SWPPP Binder |