AMENDMENTS, MODIFICATIONS AND UPDATES

In accordance with Part II C of the General VPDES Permit for Discharges of Stormwater from Construction Activities:

1. 	The operator or their designee shall amend the SWPPP whenever there is a change in the design, construction, operation, or maintenance that has a significant effect on the discharge of pollutants to surface waters and that has not been previously addressed in the SWPPP.

2. 	The SWPPP must be amended if, during inspections or investigations by the operator's qualified personnel, or by local, state, or federal officials, it is determined that the existing control measures are ineffective in minimizing pollutants in discharges from the construction activity. Revisions to the SWPPP shall include additional or modified control measures designed and implemented to correct problems identified. If approval by the VESCP authority, VSMP authority, or department is necessary for the control measure, revisions to the SWPPP shall be completed no later than seven calendar days following approval. Implementation of these additional of modified control measures must be accomplished as described in Part II H of the CGP.

3. 	The SWPPP must clearly identify the contractor(s) that will implement and maintain each control measure identified in the SWPPP. The SWPPP shall be amended to identify any new contractor that will implement and maintain a control measure.

4.	The operator or their designee shall update the SWPPP no later than seven days following any modification to its implementation. All modifications or updates to the SWPPP shall be noted and shall include the following items;

a. 	A record of dates when:

(1) Major grading activities occur;

(2) Construction activities temporarily or permanently cease on a portion of the site; and

(3) Stabilization measures are initiated.

b. 	Documentation of replaced or modified controls where periodic inspections or other information have indicated that the controls have been used inappropriately or incorrectly and where modified as soon as possible;

c. 	Areas that have reached final stabilization and where no further SWPPP inspection requirements apply;

d. 	All properties that are no longer under the legal control of the operator and the dates on which the operator no longer had legal control over each property;

e. 	The date of any prohibited discharges, the discharge volume released, and what actions were taken to minimize the impact of the release;

f. 	Measures taken to prevent the reoccurrence of any prohibited discharge; and

g. 	Measures taken to address any evidence identified as a result of an inspection.

5. 	Amendments, modifications, or updates to the SWPPP shall be signed in accordance with Part III K of the CGP

The following Amendment, Modification, and Update Log form must be used to document all amendments, modification, and updates.

SWPPP AMENDMENT, MODIFICATION AND UPDATE LOG

| Date | Description of Amendment, Modification or Update |
| --- | --- |
| _________ | ________________________________ |
| Responsible Party | _______________________________________ |
| "I certify under penalty of law that I have read and understand this document and that this document and all attachments were prepared in accordance with a system designed to assure that qualified personnel properly gathered and evaluated the information submitted. Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information, the information submitted is, to the best of my knowledge and belief, true, accurate, and complete. I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations." |  |
| ______________________________________________________ |  |
| Operator or Delegated Authority Signature |  |